General Notes to Specifier:

This Specification Section has been prepared to assist design professionals in the preparation of Project or Office Master Specifications.  It follows guidelines established by the Construction Specifications Institute, and therefore may be used with most Master Specification systems with minor editing.

Edit carefully to suit Project requirements.  Modify as necessary and delete items that are not applicable.  Verify that referenced section numbers and titles are correct.  Numbers and titles referenced are based on MasterFormat, 2004 edition.

This Section assumes the Project Manual will contain complete Division 1 documents, including Sections 01 25 13 Product Substitution Procedures, 01 33 00 – Submittal Procedures, 01 62 00 – Product Options, 01 66 00 – Product Storage and Handling Requirements, 01 7400 – Cleaning and Waste Management, 01 77 00 - Closeout Procedures, and 01 78 00 – Closeout Submittals.  Close coordination with Division 1 Sections is required.  If the Project Manual does not contain these Sections, additional information should be included under the appropriate articles.

This is an open proprietary specification allowing users the option of approving other manufacturers that comply with the criteria specified herein.

Notes to the specifier are contained in boxes and should be deleted from final copy.

Optional items requiring selection by the specifier are enclosed within brackets, e.g. [35] [40] [45].  Make appropriate selections and delete unused options.

Items requiring additional information are underlined blank spaces, e.g. _______________.

Optional paragraphs requiring selection of one of the options are separated by “OR” within a box, e.g.

OR

Bold face type identifies optional paragraphs and features that may be included or deleted depending on Project requirements.  Convert the bold face type to regular type when including these paragraphs or features.

Revise footer to suit Project/Office requirements.

Electronic versions of this specification utilize automatic paragraph numbering. When editing is complete, delete all text on this page, then remove the section break at the top of the next page to remove this page from the document.

SECTION 08210

MASTER FORMAT™ 2004 EDITION

ALUMINUM DOORS

PART 1 – GENERAL

Specifier note: information & data contained in this Guide is accurate as of June 2013.  Due to ongoing product updates, data may change over time.  Please contact your local Milgard Representative for complete product details.  Please visit milgard.com for the latest updates.

  - SUMMARY

- Section Includes:

  - Aluminum Bi-Fold Patio Doors

  - Glazing

  - Accessories

- Related Sections:

Specifier Note: Insert appropriate Section Numbers and Titles below for window and door flashing, installation sealant and painting of interior frame, sash, and divided lite grids if desired.  Windows are furnished in unfinished pine, (white) primed pine, or unfinished Douglas fir.

- __ __ __- ____________.

- __ __ __- ____________.

- __ __ __- ____________.

- __ __ __- ____________.

- __ __ __- ____________ . Finishing of interior wood frame and sash surfaces.

Specifier Note: Include appropriate language below if products specified in this Section are to be bid as alternates, otherwise delete following paragraph.

- Alternates

  - Reference Section 01 23 00 - Alternates

  - SUBMITTALS

- Reference Section 01 33 00 - Submittal Procedure; submit following items:

- Product Data: Submit Milgard’s product data.

- Shop Drawings: Include window schedule, elevations, sections, details, & multiple-window assembly details.  Include head, sill & jamb conditions; operable parts & direction/handing; and special mullion reinforcement details.

- Samples: Submit selection samples for verification, include the following:

  - Exterior Color: Minimum 1x4 color chips on aluminum substrate: (Specify)

  - Glass, showing specified tint color. (Specify)

  - Hardware: Sample of style & typical finish: (Specify)

- Quality Assurance/Control Submittals:

  - Qualifications:  Proof of Manufacturer’s qualifications.

  - U-Factor and Structural Rating charts required for NFRC and AAMA labeling requirements.

  - Installation Instructions: Follow manufacturer’s instructions.

- Closeout Submittals: Reference Section 01 78 00 – Submit following items:

  - Temporary window labels to identify windows that labels were applied to.

  - Owner’s Manual/Maintenance Instructions.

  - Special Warranties.

  - QUALITY ASSURANCE

- Overall Standards: Comply with ANSI/AAMA/101/I.S.2, except where noted herein.

- Manufacturer Qualifications:

- Minimum 10 years experience in producing aluminum doors.

- Member AAMA & NFRC.

- Regulatory Requirements and Approvals:

Specifier Note: Insert local regulatory requirements

- Certifications for Insulated Glass Units:

- Insulated glass units are certified to ASTM E2188/E2190 per the Associated Laboratories Incorporated (ALI) guidelines.

- AAMA:  Windows shall be Silver Label certified with label attached to frame per AAMA requirements.

- NFRC:  Windows shall be NFRC certified with temporary U-factor label applied to glass and an NFRC tab added to permanent AAMA frame label.

  - DELIVERY, STORAGE and HANDLING

- General: Reference Section 01 66 00 – Product Storage and Handling Requirements.

- Comply with Manufacturer’s/Dealer’s ordering instructions and lead time requirements to avoid construction delays.

- Delivery: Deliver materials in Manufacturer’s standard packaging for protection of product.

- Storage & Protection: Store products away from exposure to environmental conditions that may be harmful to materials.

- Store materials off ground in an upright position.  Provide cover from weather and construction activity.

- Follow Manufacturer’s instructions on label applied to units.

  - WARRANTY

- Residential Special Warranty: (Owner Occupied Single Family Residential)

  - Door with Water Barrier Sill: 2-Year Limited Warranty to original homeowner.

  - Insulated Glass Unit: 10-Year Limited Warranty to original homeowner.

  - Warranty doors against defects in materials and workmanship including costs for replacement parts.

  - Submit, for Owner’s acceptance, Manufacturer’s Limited Warranty document.

Part 2 - PRODUCTS

  - MANUFACTURER

- Moving Glass Wall Systems products supplied by the following manufacturer:

Milgard Manufacturing

1010 54th Avenue East

Tacoma, WA  98424

(800)-Milgard (645-4273)

milgard.com

Manufacturer’s Representative:

Specifier Note: Insert name, address and phone numbers of Manufacturer’s Representative below

Name: _____________________________________________

Telephone: __________________________________________

Fax: ________________________________________________

E-mail: _____________________________________________

- Door Series:  Milgard Moving Glass Wall Systems

- Substitutions:  Reference Section 01 25 13 - Product Substitution Procedures

  - MANUFACTURED UNITS

- Proprietary Products: Solid and tubular aluminum extruded stacking doors.

  - Moving Glass Wall Systems Aluminum Bi-Fold Doors

  - Glazing

  - Accessories

  - Substitutions: No substitutions permitted.

  - MATERIALS

- Frame and Sash:

  - Solid and Tubular Extruded Aluminum.

    - Two anodized and two painted colors.

- Weatherstripping:

  - Fin seal polypropylene pile.

Specifier Note: Verify that window flashing material and installation sealant is specified in appropriate sections.

  - SYSTEM DESCRIPTION

- General Performance Requirements: Products and systems provided must be manufactured, fabricated, and installed to the following performance criteria:

  - Comply with ANSI/AAMA/NWWDA 101/I.S.2, except as noted herein

  - Performance Class: (Specify)

  - Performance Grade: (Specify)

  - U-Factor (NFRC 100): (Specify)

  - SHGC – Solar Heat Gain Coefficient (NFRC 200): (Specify)

  - OITC – Outdoor-Indoor Transmission Class (ASTM E90): (Specify)

  - STC – Sound Transmission Class (ASTM E90): (Specify)

- Structural Requirements: Products and systems provided must be capable of withstanding wind loads based on testing units representative of those indicated for Project that pass AAMA/NWWDA 101/I.S.2/NAFS, Uniform Structural Load Test:

- Design Wind Loads: Determine design wind loads, according to ASCE, Section 6, applicable to Product from basic wind speeds (MPH) at 33 feet above grade, based upon mean roof heights indicated on Elevations/Drawings

- Basic Wind Speed: (Specify)

- Importance Factor: (Specify)

- Exposure Category: (Specify)

- Wind Load Requirement: (Specify)

  - DOOR TYPES

Specifier Note: Select from the following door options based on project requirements.  Please note related Nail Fin and Hardware styles. Delete options not used.

- Bi-Fold Patio Door – W9500 Series (Aluminum);  [Block Frame]:

  - Frame:  Solid and tubular aluminum extrusion (specify):

    - Frame depth 4-1/2”.

  - Structural Class: R-PG20.

  - Sill Type (specify):

    - ¾” Flush Sill

    - 1-1/2” Water Barrier Sill

  - Hardware (specify):

    - Lever-style.

  - GLAZING

- Insulated Glass Units: ASTM E2188/E2190 per the Associated Laboratories Incorporated (ALI) Guidelines

  - Glazing Type: Dual Glaze

    - SolarBan® 70 Low-E/Clear

  - Overall IG Unit Thickness: 7/8”

  - Spacer Type: (Specify)

    - Aluminum spacer

  - Gas Filled:

    - None

  - Glass Thickness: (Specify)

    - Per Manufacturer’s Specifications

  - FABRICATION

- Milled and Squared/Mitered Joints

  - Mechanically joined corners; no visible sash assembly fasteners

  - Trim and finish corners to match adjacent surfaces

  - FINISH

- Frame and Sash Color: (Specify)

  - [Clear Anodized] [Dark Bronze Anodized] [White] [Tan]

- Hardware: [Metal finishes as supplied by Milgard]

  - Plated Metal Finishes: Black, Stainless Steel

  - SOURCE QUALITY CONTROL:

- Inspect windows in accordance with Manufacturer's Quality Control Program as required by AAMA Silver Label Certification.

PART 3- EXECUTION

  - EXAMINATION

- Examine openings in which windows will be installed.

  - Verify that framing complies with AAMA 2400 & AAMA 2410 (Flush Fin Installation).

  - Verify that fasteners in framed walls are fully driven and will not interfere with window installation.

- Coordinate with responsible entity to correct unsatisfactory conditions.

- Commencement of work by installer is acceptance of substrate conditions.

  - INSTALLATION

- Install doors in framed walls in accordance with manufacturer’s instructions.

- Do not remove temporary labels.

  - CLEANING AND FINISHING

- Reference Section 01 74 00 – Cleaning and Waste Management.

- Remove temporary labels and retain for Closeout Submittals.

- Refer to manufacturer’s care and maintenance instructions.

- Clean wood surfaces by lightly sanding with 220-grit or finer sandpaper.

- Finish Interior Wood within 30 days of receipt per the instructions included with Product.

END OF SECTION

Issue Date: March 24, 2014