3 Part Specification

Aluminum Doors

GENERAL NOTES TO SPECIFIER:

THIS SPECIFICATION SYSTEM HAS BEEN PREPARED TO ASSIST DESIGN PROFESSIONALS IN THE PREPARATION OF PROJECT OR OFFICE MASTER SPECIFICATIONS. IT FOLLOWS GUIDELINES ESTABLISHED BY THE CONSTRUCTION SPECIFICATIONS INSTITUTE, AND THEREFORE MAY BE USED WITH MOST MASTER SPECIFICATION SYSTEMS WITH MINOR EDITING.

EDIT CAREFULLY TO SUIT PROJECT REQUIREMENTS. MODIFY AS NECESSARY AND DELETE ITEMS THAT ARE NOT APPLICABLE. VERIFY THAT REFERENCED SECTION NUMBERS AND TITLES ARE CORRECT (NUMBERS AND TITLES REFERENCED ARE BASED ON MASTERFORMAT, 2004 EDITION.)

THIS SECTION ASSUMES THE PROJECT MANUAL WILL CONTAIN COMPLETE DIVISION 1 DOCUMENTS INCLUDING 01 25 13 – PRODUCT SUBSTITUTION PROCEDURES, SECTIONS 01 33 00 – SUBMITTAL PROCEDURES, 01 62 00 – PRODUCT OPTIONS, 01 66 00 – PRODUCT STORAGE AND HANDLING REQUIREMENTS, 01 74 00 – CLEANING AND WASTE MANAGEMENT, 01 77 00 – CLOSEOUT PROCEDURES, AND 01 78 00 –CLOSEOUT SUBMITTALS. CLOSE COORDINATION WITH DIVISION 1 SECTIONS IS REQUIRED. IF THE PROJECT MANUAL DOES NOT CONTAIN THESE SECTIONS, ADDITIONAL INFORMATION SHOULD BE INCLUDED UNDER THE APPROPRIATE ARTICLES.

THIS IS AN OPEN PROPRIETARY SPECIFICATION ALLOWING USERS THE OPTION OF APPROVING OTHER MANUFACTURERS THAT COMPLY WITH THE CRITERIA SPECIFIED HEREIN.

NOTES TO THE SPECIFIER ARE CONTAINED IN BOXES AND SHOULD BE DELETED FROM FINAL COPY.

OPTIONAL ITEMS REQUIRING SELECTION BY THE SPECIFIER ARE ENCLOSED WITHIN BRACKETS, E.G. [35] [40] [45]. MAKE APPROPRIATE SELECTIONS AND DELETE OTHERS.

ITEMS REQUIRING ADDITIONAL INFORMATION ARE UNDERLINED BLANK SPACES, E.G. _______________________.

OPTIONAL PARAGRAPHS REQUIRING SELECTION OF ONE OF THE OPTIONS ARE SEPARATED BY “OR” WITHIN A BOX. E.G.

BOLD FACE TYPE IDENTIFIES OPTIONAL PARAGRAPHS AND FEATURES THAT MAY BE INCUDED OR DELETED DEPENDING UPON PROJECT REQUIREMENTS. CONVERT THE BOLD FACE TYPE TO REGULAR TYPE WHEN INCLUDING THESE PARAGRAPHS OR FEATURES.

REVISE FOOTER TO SUIT PROJECT/OFFICE REQUIREMENTS.

ELECTRONIC VERSIONS OF THIS SPECIFICATION UTILIZE AUTOMATIC PARAGRAPH NUMBERING.

WHEN EDITING IS COMPLETE, DELETE ALL TEXT ON THIS PAGE, THEN REMOVE THE SECTION BREAK AT THE TOP OF THE NEXT PAGE TO REMOVE THIS PAGE FROM THE DOCUMENT.

SPECIFICATION BEGINS ON THE FOLLOWING PAGE.

3 Part Specification

Aluminum Doors

ALUMINUM DOORS – 08 32 13

With the thin lines that Migard’s Aluminum Doors provide, they are ideal for both new construction as well as replacement.  This series of products provide an economical solution while providing architectural style.

Aluminum Doors

PART 1 – GENERAL

  - SUMMARY

- Section Includes:

      - Solid and tubular aluminum extruded doors of the following type(s):

Sliding Patio Door		Series 450S, 450 & 1600

B. 	Related Sections:

INSERT APPROPRIATE SECTION NUMBERS AND TITLES BELOW FOR WINDOW FLASHING AND INSTALLATION SEALANTS.

1.	 __ __ __ - _____________________.

2.	 __ __ __ - _____________________.

3.	08 51 13 - Aluminum Windows.

INCLUDE APPROPRIATE LANGUAGE BELOW IF PRODUCTS SPECIFIED IN THIS SECTION ARE TO BE BID AS ALTERNATES.

OTHERWISE DELETE FOLLOWING PARAGRAPH.

C. 	Alternates:

1. 	Reference Section 01 23 00 – Alternates.

Aluminum Doors  (continued)

  - SUBMITTALS

- Reference Section 01 33 00 – Submittal Procedures; submit following items:

  - Product Data.

  - Shop Drawings: Include window schedule, door elevations, sections and details, and multiple door assembly details.

  - Samples:

a. 	Color samples: Minimum 1x4 inch (25x100 mm) samples of Aluminum with painted or anodized color.

b. 	Glass, showing specified tint color.

4. 	Quality Assurance/Control Submittals:

a. 	Qualifications: Proof of manufacturer's qualifications.

b. 	U-Factor and structural rating charts required for AAMA and NFRC labeling requirements.

c. 	Installation Instructions – AAMA 2400 (“Mounting Flange Installation”).

- Closeout Submittals: Reference Section 01 78 00 – Closeout Submittals; submit following items:

  - Temporary door labels marked to identify doors that labels were applied to.

  - Maintenance instructions.

  - Special Warranties.

1.03      QUALITY ASSURANCE

- Overall Standards: Comply with ANSI/AAMA 101.I.S.2, except as otherwise noted herein.

- Qualifications:

1. 		Manufacturer Qualifications:

a. 	Minimum five years experience in producing aluminum doors of the type(s) specified.

  - Member AAMA, NFRC.

INSERT LOCAL REGULATORY REQUIREMENTS BELOW.

Aluminum Doors (continued)

- Regulatory Requirements:

- Certifications for insulated glass doors:

#### AAMA: Doors shall be Gold Label certified with label attached to frame per AAMA requirements.

#### NFRC: Doors shall be NFRC certified with temporary U-factor label applied to glass and an NFRC tab added to permanent AAMA frame label.

1.04 	DELIVERY, STORAGE AND HANDLING

- Reference section 01 66 00 - Product Storage and Handling Requirements.

- Follow manufacturer’s instructions on label applied to doors.

1.05 	WARRANTY

SELECT “RESIDENTIAL” WARRANTY BELOW FOR OWNER OCCUPIED SINGE FAMILY RESIDENTIAL AND OWNER OCCUPIED CONDIMINIUM PROJECTS.  SELECT “COMMERCIAL WARRANTY FOR NON-OWNER OCCUPIED CONDOMINIUMS, COMMERCIAL, AND MULTI-FAMILY PROJECTS.

- Residential Special Warranty:

#### Lifetime guarantee to original owner.

#### Transferability:

  - Permit unlimited transfer of ownership in first ten years.

  - Upon first transfer of ownership, guarantee period shall become ten years from date of original purchase.

#### Guarantee doors against defect in materials and workmanship including costs for parts and labor.

OR

- Commercial Special Warranty:

- 10-year guarantee.

- Guarantee doors against defects in manufacturing and workmanship including costs for parts and labor.

Aluminum Doors (continued)

PART 2 – PRODUCTS

2.01 	MANUFACTURER

- Milgard Manufacturing, Inc.	Tel:	1.800.MILGARD (645-4273)

1010 54th Avenue East		(253) 922-2030

Tacoma, WA 98424 	Fax:	(253) 926-0848

Web:	milgard.com

INSERT NAME, ADDRESS AND PHONE NUMBERS OF MANUFACTURER’S REPRESENTATIVE BELOW

- Manufacturer’s Representative:

_________________________	Tel:	_______________

_________________________	Fax:	_______________

_________________________	Email:	_______________

- Door Series: Milgard Aluminum Doors

- Substitutions: Reference Section 01 25 13 – Product Substitutions Procedures.

2.02 	MATERIALS

### Aluminum:  Comply with requirements of AAMA/WDMA/CSA 101/I.S.2/A440-05, 6063-T5 temper for strength, corrosion resistance and application of required finish.

### Extruded frame members are to be .060” in thickness for structural walls.

VERIFY THAT DOOR FLASHING MATERIAL AND INSTALLATION SEALANT IS SPECIFIED IN APPROPRIATE SECTIONS.

2.03	GENERAL PERFORMANCE REQUIREMENTS:

- Thermal Performance: Comply with NFRC 100.

- Air Leakage, Water Resistance, Structural Test: Comply with ANSI/AAMA 101/I.S.2.

- Forced-Entry Resistance: Comply with CAWM 300-96.

Aluminum Doors (continued)

2.04	DOOR TYPES:

SELECT FOLLOWING DOOR TYPES AND RELATED NAIL FIN/MOUNTING STYLE BASED ON PROJECT REQUIREMENTS. DELETE DOOR TYPES NOT USED.

A.	Sliding Patio Door – [450 Series, 1 inch (25 mm) nail fin setback with stucco key] [450S Series, 1 7/16 inch (37 mm) nail fin setback with stucco key]:

1. 	Frame:

450 Series frame depth is 4 ½” (114 mm)

b.	450S Series frame depth is 4 9/16” (116 mm)

2. 	Sash: Depth of 1 5/16” (33 mm), hollow aluminum extrusion.

3. 	Sightlines: Equal sightlines between sash and fixed glass.

4. 	Performance Class:

  - 71 ½” x 103 ½” Two Panel: SD-R20.

  - 191 ½” x 95 ½” Four Panel: SD-R20.

5.  	Hardware:

a. 	Stainless steel adjustable tandem rollers, aluminum integral monorail track.

b. 	Interior and exterior pull handle colored to match, single point lock with key option.

6.  	Weatherstripping: Fin seal polypropylene pile.

B.	Sliding Patio Door  – [1600 Series, 1 1/4 inch (32 mm) nail fin setback with stucco key]:

1. 	Frame: Depth of frame is 4 ¾” (121 mm).

2. 	Sash: Depth of 1 9/16” (40 mm), hollow aluminum profile.

3. 	Sightlines: Equal sightlines between sash and fixed glass.

4. 	Performance Class:

  - 191 ½” x 119 ½” Four Panel with 2 ft transom: SD-LC25.

  - 191 ½” x 95 ½” Four Panel: SD-HC40.

  - 191 ½” x 119 ½” Four Panel: SD-C30.

  - 119 ½” x 119 ½” Two Panel: SD-HC40 (with rebar) & SD-C30 (w/out rebar).

5. 	Hardware:

a. 	Adjustable heavy duty stainless steel tandem rollers.

b. 	Single point lock with key standard with anti-slam strike.

c. 	Interior and exterior pull handles painted to match the door.

6. 	Weatherstripping: Fin seal polypropylene pile.

Aluminum Doors (continued)

2.05	GLAZING

A. 	Insulated Glass Units: ASTM E 774, Class A, 1 inch (25mm) thick overall.

1. 	Tempered Glazing Type: [Clear/Clear] [Clear/SunCoat® Low-E] [Clear/SunCoat® Low-E, argon gas filled] [Clear/SunCoatMAX™ Low-E] [Clear/SunCoatMAX™ Low-E, argon gas filled] [Clear/Hardcoat Low-E] [Clear/Hardcoat Low-E, argon gas filled].

WARM EDGE SPACERS ARE NOT AVAILABLE ON SOME UNITS

INCLUDING CERTAIN OVERSIZE UNITS.

2. 	Spacer Bar: [Warm edge steel spacer] [Aluminum box spacer].

MOST COMMON TYPES OF INSULATED UNITS ARE INCLUDED ABOVE, BUT SEVERAL OTHER TYPES INCLUDING TINTED, REFLECTIVE, OBSCURE, AND LAMINATED ARE AVAILABLE FOR SPECIAL APPLICATIONS. SELECT DESIRED TYPES FROM MILGARD WEBSITE milgard.com/architects AND SPECIFY IN LIEU OF, OR IN ADDITION, TO THE ABOVE WITH ALL NECESSARY CRITERIA SUCH AS OBSCURE PATTERNS. IF MORE THAN ONE TYPE OF GLAZING IS REQUIRED FOR THE PROJECT, BE CERTAIN THAT TYPE FOR EACH DOOR IS CLEARLY NOTED ON DRAWINGS OR IN WINDOW SCHEDULE.

2.06	DIVIDED LITE GRIDS

VERIFY THAT DESIRED GRID PATTERNS, IF ANY, ARE SHOWN ON THE DRAWING. CERTAIN GRID PATTERNS MAY NOT BE AVAILABLE WITH ONE OR THE OTHER BAR TYPES IN THE FOLLOWING PARAGRAPH - CONSULT MILGARD FOR UNUSUAL DESIGN APPLICATIONS. GRIDS ARE NOT AVAILABLE FOR SINGLE PANE GLASS DOORS.

A. 	[5/8 inch (16 mm) wide flat, grids between the glass that are color matched to frame and sash] [1-1/16 inch (27 mm) wide sculptured, grids between the glass that are color matched to frame and sash]

2.07	INSECT SCREENS

A. 	Provide sliding screen.

1. 	Screen Cloth: Charcoal colored fiberglass mesh.

2. 	Frame:

a. 	Extruded aluminum frame with adjustable rollers.

b. 	Locking handle standard.

2.08	FABRICATION

A. 	Fabricate frames and sash with mechanically joined corners. Corners are fastened with corrosion resistant screws and sealed with an acrylic sealant.

C. 	All sashes are marine glazed with flexible PVC glazing.  The sashes will need to be disassembled to replace the glazing.

Aluminum Doors (continued)

2.09	FINISHES

A.	Frame and Sash Color: [White] [Tan] Painted Exterior Finish: Equaling 0.3 mils dry film thickness to AAMA 603.8-92.

B.	Frame and Sash Color: [Bronze] [Clear] Anodized Exterior Finish:  Provide AA-C22-A32 Class II Bronze or AA-C22-A31 Class II Clear finish, minimum 0.4 mils thick, electrolytically deposited color anodized finish.

### C.	Color match screen frame to door frame and sash color.

2.10 	SOURCE QUALITY CONTROL

- Doors inspected in accordance with manufacturer’s Quality Control Program as required by AAMA Gold Label certification.

PART 3 - EXECUTION

3.01 	EXAMINATION

A. 	Examine openings in which windows will be installed.

1. 	Verify that framing complies with AAMA 2400 (“Mounting Flange Installation”).

2. 	Verify that fasteners in framed walls are fully driven and will not interfere with door installation.

B. 	Coordinate with responsible entity to correct unsatisfactory conditions.

C. 	Commencement of work by installer is acceptance of substrate conditions.

3.02 	INSTALLATION

INSTALLATION INSTRUCTIONS (AAMA 2400) ARE ADEQUATE FOR NORMAL INSTALLATION CONDITIONS IN FRAMED CONSTRUCTION. MASONRY WALLS AND UNUSUAL CONDITIONS MAY REQUIRE ADDITIONAL INFORMATION IN THIS ARTICLE.

- Install doors in framed walls in accordance with AAMA 2400 (“Mounting Flange Installation”).

- Do not remove temporary labels.

- Install insect screens on operable sash.

Aluminum Doors (continued)

3.03 	CLEANING

- Reference Section 01 74 00 – Cleaning and Waste Management.

- Remove temporary labels and retain for Closeout Submittals.

- Clean soiled surfaces and glass using a mild detergent and warm water solution with soft, clean cloths.

END OF SECTION

This specification was prepared by Milgard Manufacturing, Inc. Comments or suggestions for improvement should be addressed to Milgard at the address in Article 2.01 A.

Issue Date: October 31, 2007