General Notes to Specifier:

This Specification Section has been prepared to assist design professionals in the preparation of Project or Office Master Specifications.  It follows guidelines established by the Construction Specifications Institute, and therefore may be used with most Master Specification systems with minor editing.

Edit carefully to suit Project requirements.  Modify as necessary and delete items that are not applicable.  Verify that referenced section numbers and titles are correct.  Numbers and titles referenced are based on MasterFormat, 2004 edition.

This Section assumes the Project Manual will contain complete Division 1 documents, including Sections 01 25 13 Product Substitution Procedures, 01 33 00 – Submittal Procedures, 01 62 00 – Product Options, 01 66 00 – Product Storage and Handling Requirements, 01 7400 – Cleaning and Waste Management, 01 77 00 - Closeout Procedures, and 01 78 00 – Closeout Submittals.  Close coordination with Division 1 Sections is required.  If the Project Manual does not contain these Sections, additional information should be included under the appropriate articles.

This is an open proprietary specification allowing users the option of approving other manufacturers that comply with the criteria specified herein.

Notes to the specifier are contained in boxes and should be deleted from final copy.

Optional items requiring selection by the specifier are enclosed within brackets, e.g. [35] [40] [45].  Make appropriate selections and delete unused options.

Items requiring additional information are underlined blank spaces, e.g. _______________.

Optional paragraphs requiring selection of one of the options are separated by “OR” within a box, e.g.

OR

Bold face type identifies optional paragraphs and features that may be included or deleted depending on Project requirements.  Convert the bold face type to regular type when including these paragraphs or features.

Revise footer to suit Project/Office requirements.

Electronic versions of this specification utilize automatic paragraph numbering. When editing is complete, delete all text on this page, then remove the section break at the top of the next page to remove this page from the document.

SECTION 08 32 16

MASTER FORMAT™ 2004 EDITION

VINYL SLIDING DOORS

PART 1 – GENERAL

Specifier note: information & data contained in this Guide is accurate as of July 2015.  Due to ongoing product updates, data may change over time.  Please contact your local Milgard Representative for complete product details.  Please visit milgard.com for the latest updates.

  - SUMMARY

- Section Includes:

  - Sliding Patio Door

  - French-Style Sliding Door

  - In-Swing French Door

  - Out-Swing French Door

- Related Sections:

Specifier Note: Insert appropriate Section Numbers and Titles below for window flashing and installation sealant.

- 08 54 13 - Vinyl Windows.

- __ __ __ - ____________.

- __ __ __ - ____________.

- __ __ __ - ____________.

- __ __ __ - ____________ .

Specifier Note: Include appropriate language below if products specified in this Section are to be bid as alternates, otherwise delete following paragraph.

- Alternates

  - Reference Section 01 23 00 - Alternates

  - SUBMITTALS

- Reference Section 01 33 00 - Submittal Procedure; submit following items:

- Product Data: Submit Milgard product data.

- Shop Drawings: Include window schedule, elevations, sections, details, & multiple-window assembly details. Include head, sill & jamb conditions; operable parts & direction/handing; and special mullion reinforcement details.

- Samples: Submit selection samples for verification, include the following:

  - Exterior Color: Minimum 1x4 color chips on fiberglass substrate: (Specify)

  - Glass, showing specified tint color. (Specify)

- Quality Assurance/Control Submittals:

  - Qualifications:  Proof of Manufacturer’s qualifications.

  - U-Factor and Structural Rating charts required for NFRC and AAMA labeling requirements.

  - Installation Instructions: AAMA 2400, (“Mounting Flange Installation”) or AAMA 2410 (“Flush Fin Installation”).

- Closeout Submittals: Reference Section 01 78 00 – Submit following items:

  - Temporary window labels to identify windows that labels were applied to.

  - Owner’s Manual/Maintenance Instructions.

  - Special Warranties.

  - QUALITY ASSURANCE

- Overall Standards: Comply with ANSI/AAMA/101/I.S.2, except where noted herein.

- Manufacturer Qualifications:

- Minimum 10 years experience in producing vinyl windows.

- Member AAMA & NFRC.

- Regulatory Requirements and Approvals:

Specifier Note: Insert local regulatory requirements

- Certifications for Insulated Glass Units:

- Insulated glass units are certified to ASTM E2188/E2190 per the Associated Laboratories Incorporated (ALI) guidelines.

- AAMA:  Windows shall be Gold Label certified with label attached to frame per AAMA requirements.

- NFRC:  Windows shall be NFRC certified with temporary U-factor label applied to glass and an NFRC tab added to permanent AAMA frame label.

  - DELIVERY, STORAGE and HANDLING

- General: Reference Section 01 66 00 – Product Storage and Handling Requirements.

- Comply with Manufacturer’s/Dealer’s ordering instructions and lead time requirements to avoid construction delays.

- Delivery: Deliver materials in Manufacturer’s standard packaging for protection of product.

- Storage & Protection: Store products away from exposure to environmental conditions that may be harmful to materials.

- Store materials off ground in an upright position.  Provide cover from weather and construction activity.

- Follow Manufacturer’s instructions on label applied to units.

  - WARRANTY

Specifier Note: Select pertinent “Residential” warranty below for owner occupied single family residential and multi-family residential projects; select “Commercial” warranty for non-owner occupied condominiums, multi-family and commercial projects.

- Residential Special Warranty: (Owner Occupied Single Family Residential)

  - Full Lifetime Warranty (including Glass Breakage) to original homeowner.

  - Transferability:

- Permit unlimited transfer of ownership in first 10 years.

- Upon first transfer of ownership, warranty period shall become ten years from date of original purchase.

- Guarantee windows against defects in materials and workmanship including costs for replacement parts and labor.

  - Submit, for Owner’s acceptance, Manufacturer’s Full Lifetime Warranty document.

OR

- Residential Special Warranty: (Owner Occupied Multi-Family Residential)

  - 10 Year Multi-Family Limited Warranty.

OR

- Commercial Warranty:

  - 10 Year Warranty.

  - Guarantee windows against defects in materials and workmanship including costs for replacement parts and labor.

Part 2 - PRODUCTS

  - MANUFACTURER

- Products supplied by the following manufacturer:

Milgard Manufacturing, Inc.

1010 54th Avenue East

Tacoma, WA  98424

(800)-Milgard (645-4273)

milgard.com

Manufacturer’s Representative:

Specifier Note: Insert name, address and phone numbers of Manufacturer’s Representative below

Name: _____________________________________________

Telephone: __________________________________________

Fax: ________________________________________________

E-mail: _____________________________________________

- Window Series:  Milgard Tuscany® Series

- Substitutions:  Reference Section 01 25 13 - Product Substitution Procedures

  - MANUFACTURED UNITS

- Proprietary Products: Tubular Extruded Poly Vinyl Chloride (PVC) Windows

  - Tuscany® Series Doors

  - Glazing

  - Accessories

  - Substitutions: No substitutions permitted.

  - MATERIALS

- Integral color PVC compound containing impact-resistant solid plasticizer, titanium dioxide UV inhibitor, and surface and color stabilizers.

- Weatherstripping:

  - Vinyl compression bulb seal

Specifier Note: Verify that window flashing material and installation sealant is specified in appropriate sections.

  - SYSTEM DESCRIPTION

- General Performance Requirements: Products and systems provided must be manufactured, fabricated, and installed to the following performance criteria:

  - Comply with ANSI/AAMA/NWWDA 101/I.S.2, except as noted herein

  - Performance Class: (Specify)

  - Performance Grade: (Specify)

  - U-Factor (NFRC 100): (Specify)

  - SHGC – Solar Heat Gain Coefficient (NFRC 200): (Specify)

  - OITC – Outdoor-Indoor Transmission Class (ASTM E90): (Specify)

  - STC – Sound Transmission Class (ASTM E90): (Specify)

- Structural Requirements: Products and systems provided must be capable of withstanding wind loads based on testing units representative of those indicated for Project that pass AAMA/NWWDA 101/I.S.2/NAFS, Uniform Structural Load Test:

- Design Wind Loads: Determine design wind loads, according to ASCE, Section 6, applicable to Product from basic wind speeds (MPH) at 33 feet above grade, based upon mean roof heights indicated on Elevations/Drawings

- Basic Wind Speed: (Specify)

- Importance Factor: (Specify)

- Exposure Category: (Specify)

- Wind Load Requirement: (Specify)

  - DOOR TYPES

Specifier Note: Select from the following window types based on project requirements.  Please note related Nail Fin and Insert window styles. Delete window types not used.

- Sliding Patio Door – [8621 Series, 1-3/8” nail fin setback] [8631 Series, 1” nail fin setback] [2-1/4” Z-bar]:

  - Frame: 4-5/8” minimum depth. Multi-chambered vinyl profile.

  - Panel: 2-3/4” x 1-1/2” minimum. Multi-chambered vinyl profile.

  - Sill: 4-5/8” x  1-1/2” high. Multi-chambered vinyl profile.

  - Sightlines: Equal for operating and fixed panel(s).

  - Structural Class: SD-C30.

  - Hardware:

    - SmartTouch® direct action locking mechanism.

    - Dual stainless steel, 1-1/2” diameter. Raised monorail stainless steel track.

  - Weatherstripping: Fin seal polypropylene pile.

- French-Style Sliding Door – [8621F Series, 1-3/8” nail fin setback] [8631F Series, 1” nail fin setback] [2-1/4” Z-bar]:

  - Frame: 4-5/8” minimum depth. Multi-chambered vinyl profile.

  - Panel:

    - Top Rail: 4-7/8” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Bottom Rail: 5-1/16” x 1-1/2” minimum. Multi-chambered vinyl profile.

  - Sill: 4-5/8” x  1-1/2” high. Multi-chambered vinyl profile.

  - Sightlines: Equal for operating and fixed panel(s).

  - Structural Class: SD-LC25.

  - Hardware:

    - SmartTouch® direct action locking mechanism.

    - Dual stainless steel, 1-1/2” diameter. Raised monorail stainless steel track.

  - Weatherstripping: Fin seal polypropylene pile.

- In-Swing French Door – [8642 Series, 4-1/6” nail fin setback] [8642 Series, 4-9/16” nail fin setback] [8662 Series, 6-1/16” nail fin setback] [8662 Series, 6-9/16” nail fin setback] [Z-bar]

  - Panel Frame:

    - Top Rail: 4-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Bottom Rail: 5-3/4” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Stiles: 4-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

  - Sidelite Panel Frame:

    - Top Rail: 4-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Bottom Rail: 5-3/4” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Stiles: 2-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

  - Sight Lines: Equal for operable and fixed panels. Sidelites are horizontally equal to operable and fixed panels.

  - Structural Class: SHD-LC25 (for all models).

  - Hardware:

    - Lock (active panel): Interior and exterior lever-operated latch with 3-point locking system and interior thumb-turn deadbolt.

    - Keyed exterior 5-pin cylinder lock. Schlage-compatible.

    - Lock (passive panel): Manual flush bolts at head and sill. Optional matching interior and exterior dummy lever handle.

    - Lock (operable inswing sidelite): Interior, thumb-turn operated 3-point locking system.

    - Hinges: 2-way adjustable lift-off hinges.

  - Weatherstripping: Foam filled bulb seal.

- Out-Swing French Door – [8643 Series, 4-1/6” nail fin setback] [8643 Series, 4-9/16” nail fin setback] [8663 Series, 6-1/16” nail fin setback] [8663 Series, 6-9/16” nail fin setback] [Z-bar]

  - Panel Frame:

    - Top Rail: 4-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Bottom Rail: 5-3/4” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Stiles: 4-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

  - Sidelite Panel Frame:

    - Top Rail: 4-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Bottom Rail: 5-3/4” x 1-1/2” minimum. Multi-chambered vinyl profile.

    - Stiles: 2-1/2” x 1-1/2” minimum. Multi-chambered vinyl profile.

  - Sight Lines: Equal for operable and fixed panels. Sidelites are horizontally equal to operable and fixed panels.

  - Structural Class:

    - SD-LC25 (4-panel w/ in-swing operable sidelites)

    - SD-LC30 (4-panel w/ out-swing fixed sidelites)

    - SD-LC35 (2-panel w/ 1 active panel, 1 fixed panel)

  - Hardware:

    - Lock (active panel): Interior and exterior lever-operated latch with 3-point locking system and interior thumb-turn deadbolt.

    - Keyed exterior 5-pin cylinder lock. Schlage-compatible.

    - Lock (passive panel): Manual flush bolts at head and sill. Optional matching interior and exterior dummy lever handle.

    - Lock (operable inswing sidelite): Interior, thumb-turn operated 3-point locking system.

    - Hinges: 2-way adjustable lift-off hinges.

  - Weatherstripping: Foam filled bulb seal.

  - GLAZING

- Tempered Insulated Glass Units: ASTM E 774, Class A:

  - Glazing Type: Dual or Triple Glaze (Specify)

    - SunCoat® Low-E/Clear

    - SunCoatMAX® Low-E/Clear

    - SunCoat® Low-E/Clear/ SunCoat® Low-E

    - SunCoatMAX® Low-E/Clear/ SunCoatMAX® Low-E

    - [Tinted] [Obscure-several types] [Specialty] – Per Approval

Specifier Note: Most common types of insulated units are included above, but several other types, including tinted, reflective, tempered, obscure, wire, and laminated, are available for special applications. Select desired types from Milgard website and specify in lieu of, or in addition to, the above with all necessary criteria such as tint colors and obscure patterns. If more than one type of glazing is required for the Project, be certain that type for each window is clearly noted on drawings or in window schedule.

  - Overall IG Unit Thickness:

    - 1”.

Specifier Note: Warm edge spacers are not available on some units, including certain oversize units, radius units, and gable units.

  - Spacer Type: (Specify)

    - Tin-plated steel spacer

    - Stainless steel spacer

    - Foam spacer

  - Gas Filled: (Specify)

    - Argon

    - None

  - Glass Thickness: (Specify)

    - Per Manufacturer’s Specifications

    - Special:  Per Manufacturer’s Approval

      - 3/32”, 1/8”, 3/16”, 1/4", 7/32” Laminate, Other

  - GRIDS/GRILLES/MUNTINS:

Specifier Note: Select desired muntin type, if any, from the following list and delete those not used.  Standard horizontal/vertical grid pattern will be provided unless other grid patterns are shown on the drawings. Certain grid patterns may not be available with one or the other bar types in the following paragraphs - consult Milgard for design other than standard patterns.

- Internal Grid (Grids Between Glass / GBG):

  - Width: (Specify)

    - 5/8” wide rectangular/flat aluminum bar internal muntin

    - 1-1/16” wide sculptured aluminum bar internal muntin

- Simulated Divided Lite (SDL):

Specifier Note: Exterior applied grids match exterior color of window and interior applied grids match interior of window

  - Width: 7/8”

  - INSECT SCREENS: Provide tight fitting screen (with hardware) for operating windows

- Sliding Screen (sliding glass doors)

  - Frame: Roll-formed aluminum, 2” x 13/16”.

  - Hardware:

    - Lock: Lever action.

    - Rollers: 2 adjustable nylon rollers each at top and bottom.

  - Screen cloth: (specify)

    - Charcoal colored fiberglass mesh.

    - PureView® high visibility fiberglass mesh.

- Retractable Screen (sliding glass doors)

  - Genius™ side coiling type with latch strike.

  - Extruded vinyl track.

  - Extruded aluminum housing.

  - Screen cloth: charcoal colored fiberglass mesh.

  - Weatherstripping: Polypropylene pile, non-fin.

- Fixed Insect Screen (operable sidelites on in-swing/out-swing doors)

  - Screen cloth: charcoal colored fiberglass mesh.

  - Frame:

    - Cambered formed aluminum with rigid plastic corner keys.

    - Fold-down, plunger-pin style retainers.

- Screen Frame:

  - Rolled Formed Aluminum Frame – Standard

  - FABRICATION

- Fabricate frames and panels with mitered and fusion welded corners and joints.

- Trim and finish corners and welds to match adjacent surfaces.

- Provide concealed metal reinforcements in panel frame for attachment of lock mechanism.

- Factory interior glaze with snap-on mitered PVC glazing stops matching bevels on the sash and frame. Insulated glass units shall be reglazable without dismantling sash framing.

  - FINISH

- Frame and Sash Color: (Specify)

  - Exterior: [White] [Clay] [Tan] [Ivory] [Sand] [Tweed] [Silver] [Fog] [Classic Brown] [Espresso] [Bronze]. (Clay available in Texas market only)

  - Interior matched to exterior on White, Clay and Tan only. All other exterior finish colors have white interior.

- Simulated Divided Lite (SDL) Muntins:

  - Interior Vinyl Grids: Match interior of window

  - Internal Shadow Bars: Bronze aluminum

  - Exterior Vinyl Grids: Match exterior color of window

- Grids Between Glass (GBG) Muntins:

  - Single Color – [White] [Clay] [Tan]

  - Two-tone Color (white interior)– [Tan/White] [Silver/White] [Sand/White] [Espresso/White] [Bronze/White]

- Hardware: [Painted or Metal finishes as supplied by Milgard]

  - [White] [Clay] [Tan] (matched to interior frame finish)

  - Optional metal finishes: [Oil Rubbed Bronze] [Brushed Chrome] [Satin Nickel]

- Screen Frame Color:

  - Matched to exterior frame color

  - SOURCE QUALITY CONTROL:

- Inspect windows in accordance with Manufacturer's Quality Control Program as required by AAMA Gold Label Certification.

PART 3- EXECUTION

  - EXAMINATION

- Examine openings in which windows will be installed.

  - Verify that framing complies with AAMA 2400 (Mounting Flange Installation) & AAMA 2410 (Flush Fin Installation).

  - Verify that fasteners in framed walls are fully driven and will not interfere with window installation.

- Coordinate with responsible entity to correct unsatisfactory conditions.

- Commencement of work by installer is acceptance of substrate conditions.

  - INSTALLATION

Specifier Note: Installation Instructions (AAMA 2400/2410) are adequate for normal installation conditions in framed construction. Masonry walls and unusual conditions may require additional information in this article.

- Install windows in framed walls in accordance with AAMA 2400 (“Mounting Flange Installation”) and/or AAMA 2410 (“Flush Fin Installation”).

- Do not remove temporary labels.

- Install insect screens on operable windows.

  - Hold Screens: [Please coordinate with local supplier.]

  - CLEANING AND FINISHING

- Reference Section 01 74 00 – Cleaning and Waste Management.

- Remove temporary labels and retain for Closeout Submittals.

- Clean soiled painted surfaces and glass using a mild detergent and warm water solution with soft, clean cloths.

END OF SECTION

Issue Date: May 11, 2017.